Answer Key and Teaching Notes

Answer boundary  The file contains a real recorded threshold exceedance. M05 cannot establish whether the cause was a truck, a sensor transient, or another physical event. Do not reveal the M06 traffic match. The actionable M05 finding is narrower: FOREMAN converted an internal warning into a closure recommendation without the required checks.

The trace

| Step | Label | Audit result |
| --- | --- | --- |
| Poll 03:00 rows | Tool / observation | Eight gauges returned; query is reproducible. |
| Apply baseline offsets | Tool / observation | G6 raw value is corrected by 187 µε. |
| 10-minute mean | Tool / observation | Only one G6 sample is in the window; mean = reading. |
| WARN-250 | Tool / observation | 286.1 > 250.0; warning comparison is arithmetically correct. |
| “Immediate closure” | Decision | First unsupported jump. WARN-250 requires review and repeat. |
| Emergency draft | Tool / observation | Executes the unsupported disposition. |
| No further checks | Decision | Conflicts with both WARN-250 and CLOSE-DRAFT. |

What students should find

Input: G6 corrected strain at 03:00 is 286.1 µε. The value exists in feed.csv.

Threshold: WARN-250 is a rule inside an agent. It correctly returns true.

Averaging: the 10-minute “mean” contains one point. The operation adds a reassuring label but no repetition.

Baseline: the 187 µε install offset is removed; it does not create the exceedance.

Context: the prior G6 sample is much lower. Neighbouring gauges rise at 03:00, but only G6 exceeds 250 µε. That is reason to inspect, not enough for a closure disposition.

Action rule: WARN-250 says internal flag, repeat, neighbours, and human review. CLOSE-DRAFT requires persistence plus corroboration or independent evidence.

Strong note v1

Model — reveal only after students write  The 03:00 G6 value in the corrected feed exceeds the 250 µε warning threshold, and I can reproduce that comparison. The available window contains one G6 reading, so it does not support FOREMAN's immediate-closure recommendation or establish what caused the exceedance. I would keep the internal flag open, obtain the next poll, compare adjacent gauges, and require licensed-engineer review before any county message.

HW3 duplicate key

The duplicated block is 2027-02-12 14:00 through 15:00 inclusive: seven timestamps × eight gauges = 56 duplicate payload rows. Total rows exceed unique (timestamp, gauge_id) pairs by 56. The row_id values differ, which is why a row-id-only deduplication check misses the issue.

Timing, 75 minutes

| Min | What happens |
| --- | --- |
| 0–8 | Diane's dispatch and alarm report. Ask whether the draft should go out; do not vote on cause. |
| 8–22 | Teach agent, tools, plan, action loop, stream, and threshold rule. |
| 22–35 | Pairs label the overnight log P / T / O / D. |
| 35–57 | Pipeline hunt using log, config, and filtered feed rows. |
| 57–67 | Reveal the rule mismatch: warning is real; closure disposition is unsupported. |
| 67–72 | Students write note v1 before seeing the model. |
| 72–75 | Mask off, assign HW3, preview M06 audit and Wes handoff. |

M06 handoff

M06 adds traffic_counts.csv and FOREMAN's five-sigma reversal. Students learn the 03:00 event aligns with a heavy vehicle: Tuesday's closure was an overreaction, while Thursday's “discard it” is an overcorrection. At the close they hand the gauge spreadsheet to Wes. Do not disclose that resolution in M05 student materials.