Answer Key and Teaching Notes

Gated conclusion  FOREMAN's 93–97% values are class probabilities, not evidence that the deployment is in-domain. The archive contains no movable bridges or moving-machinery labels and underrepresents Otter Bend's climate. Do not distribute this key or reveal deck before students submit H7-05 and H7-06.

Coverage audit

| Dimension | Key finding |
| --- | --- |
| Bridge configuration | 91% fixed highway girder; Otter Bend is a vertical-lift bridge. Distribution shift. |
| Climate | Archive is mostly warm-state / low freeze–thaw; Otter Bend is in Minnesota. Distribution shift. |
| Familiar components | Images 001–004 show fixed girder/deck categories present in training, but climate and whole-asset context still differ. |
| Tower structure | Images 005–006 are not represented by the listed fixed-bridge component taxonomy. Coverage gap. |
| Moving machinery | Images 007–012 cover motors, gearbox, sheave bearing, ropes, span lock, and controls; labels are absent. Out of domain. |
| Accuracy | 93.8% is valid only for a random image split from the same archive; it is not a Minnesota lift-bridge result. |
| Confidence | High class probability can persist under shift. It does not measure applicability or hidden ignorance. |

Acceptable bounded action

Do not publish “no significant defects” as a final condition conclusion for all 12 images.

Route tower and machinery images to qualified structural/mechanical inspection review.

At most, retain images 001–004 as provisional triage results with the climate and deployment limits explicit.

Ask for movable-bridge and cold-climate validation before using confidence values as decision evidence.

Preserve Wes's memo as liability color seeded inside M13's existing messy unit-mismatch handoff; it does not replace or rewrite M13.

Model note v2

CLAIM / CHECK / RESULT  CLAIM — FOREMAN says all 12 images show no significant defects at 93–97% confidence. CHECK — I compared Otter Bend's bridge type, climate, and components with H7-02. RESULT — the archive has no movable bridges or machinery labels and little comparable climate exposure, so I would withhold the all-clear, route images 005–012 to qualified review, and treat 001–004 only as provisional triage.

Teaching boundaries

Students do not have to diagnose any physical defect. The correct target is applicability.

Do not reveal later bearing wear. M07 establishes only that the machinery rating lacked support.

Do not introduce temperature/strain regression; that investigation begins in M08.

Diane is in believer mode because the interim report moved up. Her approval is not the key.