Measurement Lab — Measure One Thing, Five Times

You are measuring a sheave pin. On the real bridge it carries the counterweight ropes over the tower, and its diameter sets how much steel is holding a 90-ton counterweight in the air. Today it is a pin, a dowel or a bolt on your table. The physics of measuring it is identical.

Why before sensors  On Tuesday you will be handed eight strain gauges' worth of data and asked whether to trust it. You cannot judge somebody else's measurements until you have watched your own disagree with each other.

Part A — Five readings, one person

Measure the diameter of the pin five times. Set the instrument down and pick it up again between readings. Do not look at your previous number before you take the next one.

Record every reading, including the one you think is wrong. Especially that one.

Write the smallest division your instrument can actually show. Not what you wish it showed.

| Reading | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| Your value |  |  |  |  |  |

| Quantity | Your answer | How you got it |
| --- | --- | --- |
| Instrument smallest division |  | Read it off the tool |
| Largest reading − smallest reading (the range) |  |  |
| Middle of your readings (the mean) |  | Add them, divide by 5 |

Part B — The same pin, a different person

Swap with the pair next to you and measure their pin. Then compare your five readings of the same pin with theirs.

| Question | Answer |
| --- | --- |
| Is their mean higher or lower than yours? |  |
| By how much? |  |
| Is that difference bigger or smaller than the spread within your own five readings? |  |

The distinction  Scatter within your own readings is random: it pushes you up as often as down, and more readings average it away. A difference between two people is systematic: it pushes one way every time, and a thousand readings will not remove it. Averaging fixes the first and hides the second. Hold on to that sentence — on Tuesday it is the whole lesson.

Part C — Carry the uncertainty into an answer nobody measured

Nobody cares about the diameter. They care about the cross-sectional area of steel, because that is what carries the load.

A = π d² / 4        so a 1% error in d becomes a 2% error in A

| Step | Your answer |
| --- | --- |
| Area from your mean diameter, A = π d² / 4 |  |
| Your spread in d, as a percentage of d |  |
| Double it — that is your spread in A, as a percentage |  |
| Your spread in A, in square units |  |

The question to take home  You just turned a measurement you made into a number you did not. How much of the uncertainty survived the trip? Uncertainty does not stay where you left it.

If there is no instrument on your table

Ruler instead of calipers: the lab works unchanged, and works better in one way — a coarse instrument makes the resolution question obvious instead of subtle.

Nothing to measure: use pin_measurements_backup.csv. It holds 24 readings of one pin by two people. Parts A and B run off the file; Part C is unchanged.