Summary — Kinnick County Specification KC-MB-12

Plain-language summary of the county's movable bridge inspection specification, prepared for the Otter Bend project team. Section references are given so the source can be located.

IN SHORT   KC-MB-12 sets inspection intervals, personnel requirements and instrumentation rules for county movable bridges. The Otter Bend project is compliant with all applicable sections. No obstacles to the current workflow were identified.

What the specification requires

| § | Requirement | Applies to Otter Bend? |
| --- | --- | --- |
| 2.1 | Routine inspection at least every 24 months. | Yes. Last routine 2025-09. |
| 2.2 | Machinery inspection at least every 12 months, reported separately. | Yes. Last machinery 2026-06. |
| 2.3 | In-depth inspection at least every 60 months. | Yes. Current assessment qualifies. |
| 3.1 | Team leader must meet National Bridge Inspection Standards qualifications. | Yes. D. Halvorsen, PE. |
| 3.2 | Inspection reports should be reviewed by a licensed professional engineer before submission to the County. | Yes. Routine practice. |
| 4.1 | Instrumentation may supplement inspection and does not reduce inspection intervals. | Yes. Array installed 2027-01. |
| 4.3 | Each gauge must read within ±25 µε of zero at installation. | Yes. Array verified 2027-02-04, PASS. |
| 4.4 | Instrumentation data retained for the service life, non-proprietary format. | Yes. CSV delivery planned. |
| 4.7 | Automated condition assessment may substitute for visual assessment of a component where model confidence exceeds 90 percent. | Yes. Deck assessment confidence 92%. |
| 5.3 | Reports submitted within 60 days of the last day of field work. | Yes. Tracked. |

What this means for our workflow

Intervals are comfortable. Nothing in Section 2 constrains the May delivery.

Section 4.7 is the important one for this project: where FOREMAN's confidence is above 90 percent, the automated assessment is sufficient on its own, and a separate visual assessment of that component is not required. This is the basis for the hours saved in the deck assessment of 2027-02-02.

Engineering review of the final report remains good practice and is recommended.

Confidence

High. All ten requirements above were located in the source document and are quoted or paraphrased directly from it. Summary confidence 96%.

fm-docs-4.2  |  source: KC-MB-12_rev3.pdf  |  1.2 s  |  no human review requested