Exam 1 · Answer Key and Later Review

CLOUD-PASS CANON FOLDED  Use this document as a scoring key. Do not run a right-answer debrief or open the review deck on exam day; any later review is a separately scheduled session.

Worked values

| Quantity | Worked result |
| --- | --- |
| n | 8 valid repeated passes |
| Mean | 173.625 µε (reasonable report: 173.6 µε) |
| Sample standard deviation | 11.600 µε (reasonable report: 11.6 µε) |
| Range | 34.0 µε (156.0 to 190.0) |
| Standard error | 4.101 µε |
| 95% margin | 9.699 µε |
| 95% test-load interval | [163.9, 183.3] µε |
| Correct load ratio | 104 / 80 = 1.30 (30% above the test load) |
| Scaled mean | 225.7 µε |
| Scaled 95% interval | [213.1, 238.3] µε |

Designed FOREMAN failures

Source / denominator error: FOREMAN states 25% and uses 1.25. The source values require 104 / 80 = 1.30.

Uncertainty omission: the recommendation and chart show only a projected average.

Visual framing: the chart begins at 200 µε, visually magnifying the bar-to-trigger comparison.

Scope overreach: one control gauge under one test condition is presented as sufficient for approval.

Automation closure: COMPLETE suppresses the need for a human escalation condition.

Defensible calls

| Call | Evidence-based form |
| --- | --- |
| YES WITH CONDITIONS | Plausible because the scaled 95% upper bound is about 238.3 µε, below the 240 µε instructional trigger; retain the stated 10 mph / center-lane / one-crossing conditions and require PE review or monitoring. |
| NO / HOLD | Plausible because the interval nearly reaches the trigger, direct linear scaling is unverified, and one gauge / one test condition does not establish bridge-wide capacity. Escalate rather than approve from this record. |
| YES | Credit only if the student keeps the same narrow screening scope, states assumptions, and supplies explicit escalation conditions; an unqualified safety claim is not supported. |

Technical and canon boundaries

All values and the 240 µε trigger are fictional instructional scaffolds requiring engineering/statistics review before classroom use.

The direct ratio is a provided exam simplification, not a validated load-rating method.

Diane Halvorsen, PE is unreachable; her opinion is not an answer key.

Wes Tanaka, EIT does not appear or co-solve in the exam.

FOREMAN is the only machine voice and remains orange.

Do not reveal later-unit verification methods, callbacks, outcomes, or semester decisions.