Simplified Load Rating — Approach Span Girders

ACMEJOB.Ai standard sheet LR-1. Use for interior girders of the Otter Bend north and south approach spans only. This is a teaching simplification of the operating rating procedure; it is not a substitute for the governing rating manual.

The rule

RF  =  ( C − D ) ÷ ( L × IM )

Rating (tons)  =  RF × 36

| Symbol | Means | Where it comes from |
| --- | --- | --- |
| C | Girder moment capacity, kip-ft | Section properties + material tests |
| D | Dead load moment, kip-ft | Weight of the structure itself |
| L | Live load moment, kip-ft | The rating truck, here an HS-20 at 36 tons |
| IM | Impact factor (dimensionless) | Allowance for a moving load bouncing. 1.33 here. |
| RF | Rating factor | Above 1.00 the girder carries the rating truck. |

Worked example — Girder G-3, south approach

| Step | Substitution | Result |
| --- | --- | --- |
| 1. Capacity minus dead load | 3,920 − 1,455 | 2,465 kip-ft |
| 2. Live load with impact | 1,080 × 1.33 | 1,436.4 kip-ft |
| 3. Rating factor | 2,465 ÷ 1,436.4 | 1.716 |
| 4. Rating in tons | 1.716 × 36 | 61.8 tons |

Reading it  RF is 1.716, which is above 1.00, so G-3 carries the rating truck with room to spare. Every step above can be checked by someone who was not in the room. That is the whole point of a rule.

Your turn — Girder G-4, south approach

G-4 sits under the wheel line and its bottom flange has section loss from 60 years of runoff, so its capacity is lower. Dead and live load are unchanged.

| Input | Value |
| --- | --- |
| C, capacity (reduced for section loss) | 3,850 kip-ft |
| D, dead load moment | 1,420 kip-ft |
| L, live load moment | 1,080 kip-ft |
| IM, impact factor | 1.33 |

| Step | Show your substitution | Result |
| --- | --- | --- |
| 1. C − D |  |  |
| 2. L × IM |  |  |
| 3. RF = step 1 ÷ step 2 |  |  |
| 4. Rating = RF × 36 |  |  |

Before you move on  Is your RF above or below 1.00? Say in one sentence what that means for a 36-ton truck crossing the south approach, and what you would want to know before saying it out loud to the county.