Sensor Installation Record — Strain Gauge Array

| Field | Entry |
| --- | --- |
| Structure | Otter Bend Lift Bridge, County Road 9 over Kinnick River |
| Installed | January 14, 2027 |
| Installed by | Northline Instrumentation, subcontract 27-114-S2 |
| Array | 8 bonded foil strain gauges, G1 through G8, midspan and quarter points |
| Instrument resolution | 1 microstrain (µε) |
| Witnessed by | W. Tanaka, EIT |

Acceptance check

Specification KC-MB-12 §4.3 requires that under no load, each gauge read within ±25 µε of zero. Twelve readings were taken at each gauge with the span down and the roadway closed. Raw readings are in gauges_install.csv.

| Gauge | Location | Installed | Notes from the field |
| --- | --- | --- | --- |
| G1 | S approach, midspan | 2027-01-14 |  |
| G2 | S approach, quarter pt | 2027-01-14 |  |
| G3 | S tower base | 2027-01-14 |  |
| G4 | Lift span, quarter pt | 2027-01-14 |  |
| G5 | Lift span, midspan | 2027-01-14 |  |
| G6 | Lift span, midspan (redundant) | 2027-01-14 | Re-bonded after first attempt lifted |
| G7 | N tower base | 2027-01-14 |  |
| G8 | N approach, midspan | 2027-01-14 |  |

Signed off  Array installed and accepted. Northline's technician reported the array within specification and left the site at 14:20. No individual gauge values were recorded on this form.

Northline guy was in a hurry. I watched him re-glue G6 and didn't see him re-zero it. Said it was fine. — W.T.

Handwritten in the margin of the original. Nobody followed up.