KC-MB-12 — Movable Bridge Inspection Specification

Revision 3  ·  Adopted March 2024  ·  Supersedes Rev. 2 (2016)  ·  Applies to all movable bridges owned or maintained by Kinnick County

1   Scope and Application

1.1  This specification governs the inspection of movable bridges owned or maintained by Kinnick County, including vertical lift, bascule and swing spans. It applies to the structure, the operating machinery, and any instrumentation installed on either.

1.2  Where this specification and a state or federal requirement differ, the more frequent inspection interval and the more restrictive personnel requirement govern.

1.3  In this specification, “movable span” means the portion of the structure that is raised, rotated or otherwise displaced to pass navigation. “Machinery” means the drive motors, reducers, shafts, bearings, sheaves, ropes, counterweights and span locks.

2   Inspection Types and Intervals

2.1  Routine inspection. Every movable bridge shall receive a routine inspection at intervals not exceeding 24 months.

2.2  Machinery inspection. Operating machinery shall be inspected at intervals not exceeding 12 months. A machinery inspection may be performed concurrently with a routine inspection but shall be reported separately.

2.3  In-depth inspection. An in-depth inspection shall be performed at intervals not exceeding 60 months, or whenever a routine inspection identifies a condition the team leader cannot assess from the routine inspection alone.

2.4  Special inspection. A special inspection shall be performed within 30 days of any of the following: a vessel or vehicle impact; a flood exceeding the 10-year stage; a load exceeding the posted limit; a failure of a machinery component; or written direction from the County Engineer.

3   Personnel and Responsible Charge

3.1  Each inspection shall be performed by a team under the direction of a team leader meeting the qualification requirements of the National Bridge Inspection Standards.

3.2  A licensed professional engineer shall review and seal every inspection report before it is submitted to the County. The sealing engineer shall be in responsible charge of the findings, regardless of what tools, software or automated methods were used to produce them.

3.3  Machinery inspection shall include at least one person with documented experience on movable bridge machinery of the same type.

4   Instrumentation and Monitoring

4.1  Instrumentation may be installed to supplement inspection. Instrumentation does not reduce any inspection interval in Section 2.

4.2  Permitted instrumentation includes strain gauges, displacement transducers, accelerometers, thermocouples and tilt sensors.

4.3  Baseline acceptance. Following installation, each gauge shall be verified under no load. Each gauge shall read within ±25 µε of zero. A gauge failing this check shall be re-zeroed or replaced before the array is accepted.

4.4  Instrumentation data shall be retained for the service life of the structure and delivered to the County in a non-proprietary format.

4.5  Reporting. Instrumentation data submitted to the County shall identify the instrument, its resolution, its calibration date, and any processing applied to the raw readings.

5   Documentation and Records

5.1  Each inspection report shall contain: the date and type of inspection; the team leader and team members; condition ratings with the basis for each; photographs keyed to their locations; any recommended action with a recommended date; and the seal required by 3.2.

5.2  Reports shall be retained by the County for the service life of the structure.

5.3  Reports shall be submitted within 60 days of the final day of field work.

End of specification KC-MB-12 Rev. 3. This document contains Sections 1 through 5 in full. There are no appendices.