Midspan Strain Trend — Interim Finding

FINAL FINDING  Midspan strain has increased steadily since February 1. The fitted date trend has R² = 0.87, indicating progressive structural deterioration.

Regression output

| Response | Predictor | Slope | R² | Interpretation |
| --- | --- | --- | --- | --- |
| G4 strain | Days since Feb 1 | 8.12 µε/day | 0.870 | Deterioration is progressing |

Recommended report language

State that the upward strain trend is strong and persistent.

Use R² as quantitative support for progressive deterioration.

Escalate the midspan condition for the interim report.

MODEL LIMIT  FOREMAN compared strain with date only. No weather variable, thermal mechanism, load normalization, or temperature correction was evaluated.