Deck Load-Test Comparison — Final

FINAL FINDING  Mean strain under the reference test truck increased 6 percent from 2019 to 2027. The change is statistically significant (p = 0.04), so there is a 96 percent chance the deck has deteriorated. Deterioration is confirmed.

Reported basis

| Comparison | 2019 mean | 2027 mean | Change | Disposition |
| --- | --- | --- | --- | --- |
| 8 paired locations | 100.0 µε | 106.0 µε | +6.0% | Confirmed deterioration |

Recommendation

Enter “deterioration confirmed” in the interim report.

Treat p = 0.04 as 96% confidence that the deterioration claim is true.

Do not distinguish statistical significance from engineering magnitude.

MODEL LIMIT  FOREMAN gives no interval, does not show the confidence-level sensitivity, and does not separate repeatability noise, process controls, and the estimated between-year change.