Paired Load-Test Evidence Guide

The comparison uses the same eight instrumented locations and three matched truck passes per location in each year. Analyze location means as eight paired observations.

| File / field | Meaning | Unit / role |
| --- | --- | --- |
| loadtest_2019_2027.csv | 48 pass-level records: 8 locations × 2 years × 3 passes | source |
| analysis_strain_microstrain | temperature-corrected response under the reference truck | µε |
| reference_load_kips | matched reference test load | 80.0 kips |
| lane_offset_ft / speed_mph | matched process controls | 0.0 ft / 5.0 mph |
| M10-paired-location-summary.csv | year means, paired change, and repeatability SD | analysis |

Noise audit

| Source | How M10 checks or limits it | What remains |
| --- | --- | --- |
| Measurement repeatability | Three passes; within-test SD reported |  |
| Location-to-location variation | Same-location pairing |  |
| Truck load / lane / speed | Matched controls in both years |  |
| Temperature response | Correction already applied and recorded |  |
| Sampling uncertainty | Standard error and t interval on 8 changes |  |

Boundary  These controls make a small between-year signal more credible. They do not identify a damage mechanism or prove the whole deck deteriorated uniformly.